ESE 4330, Fall 2025

ADS Project 2 Filters

Due 11/10 (1:00 pm CT)

In this design project, you will be asked to design three low-pass filters with a cutoff frequency of 3 GHz, source impedance of 50 Ω, at least 15 dB attenuation at 4.5 GHz, and a 0.5 dB equal-ripple response. The substrate is FR4 with εr = 4.2, thickness d = 0.079 cm, tanδ = 0.02, and 0.5-mil-thick copper conductors.

Upon completion of the project, everyone needs to submit the electronic design file (ADS workspace file) and a project report.

- LPF design using lossless lumped elements. Finish the schematic in ADS, and plot insertion loss vs. frequency from dc to 6 GHz.

- LPF design using microstrip-line shunt stubs. Finish the schematic and layout in ADS, and plot insertion loss vs. frequency from dc to 6 GHz.

- LPF design using stepped-impedance method. Finish the schematic and layout in ADS, and plot insertion loss vs. frequency from dc to 6 GHz.

Report:

Your report needs to clearly document the filter design steps. You also need to explain the performance difference between Momentum simulation and schematic-level simulation, if there is any. In addition, your report needs to have a table for each filter design to summarize its performance (cutoff frequency, passband insertion loss, passband ripple, attenuation at 4.5 GHz) and benchmark with the target specifications.

Make sure that your graphs and markers are readable.  Comment on what you are doing and explain your work precisely and concisely. Should be submitted in 2-column IEEE format (https://www.ieee.org/conferences/publishing/templates.html)

One PDF per group. Save your group's ADS project workspace in a shareable link. List each member’s contributions (design, schematic, layout, sims, write-up).